4WD Bluetooth-Controlled

Project Report

DATE:

January 25, 2026

Submitted By:

Frais Asghar
CMS ID: 516900
ME-16(Section-A)

Arduino Robot Car

4WD-Bluetooth-Controlled Arduino Robot Car

## 1. Executive Summary

This report details the design, assembly, and implementation of a 4-Wheel Drive (4WD) robot car. The system leverages the Arduino Uno microcontroller for logic processing, an HC-05 module for wireless Bluetooth communication, and an L298N H-Bridge for motor drive control. The project successfully demonstrates a mobile-controlled robotics platform with high maneuverability and modular architecture for future sensor integration.

## 2. Project Objectives

The primary goals of this project are:

- To design and assemble a robust 4WD mechanical chassis capable of varying terrains.

- To implement a wireless control system using Bluetooth (HC-05) for real-time mobile interaction.

- To develop efficient motor drive logic using the L298N H-Bridge for multi-directional movement.

- To create an expandable open-source platform for learning embedded systems and robotics.

## 3. System Architecture

The robot is designed using a layered architecture:

- Control Layer: Arduino Uno (ATmega328P).

- Communication Layer: HC-05 Bluetooth Module (Serial Communication at 9600 baud).

- Power & Drive Layer: L298N Dual H-Bridge Motor Driver and 4x DC Geared Motors.

- Energy Layer: 18650 Lithium-Ion Battery pack (High Energy Density).

## 4. Hardware Implementation

### 4.1 Component Specifications & Costing

The following table outlines the components used and their approximate market pricing (PKR):

| Component | Function | Technical Detail | Approx. Price |
| --- | --- | --- | --- |
| Arduino Uno | Master Controller | 16 MHz Clock, 5V Logic | 1000-1500 PKR |
| L298N Driver | Motor Power Control | Dual channel, 2A/channel | 500-700 PKR |
| HC-05 Module | Wireless Interface | BT 2.0+EDR, Range ~10m | 700-900 PKR |
| 4WD Chassis | Mechanical Base | 4x Geared Motors + Wheels | 1800-2200 PKR |
| 18650 Batteries | Power Source | 3.7V per cell (2 Units) | 600-800 PKR |
| Battery Holder | Energy Housing | 3-cell with switch | 400-500 PKR |
| Breadboard | Prototyping | 400/800 points | 400-500 PKR |
| Jumper Wires | Connectivity | M-M, M-F, F-F | 100-200 PKR |
| Total Estimated Cost |  |  | 5500 - 7300 PKR |

### 4.2 Circuit Connectivity

The integration follows a specific pin-mapping strategy to allow for PWM (Pulse Width Modulation) speed control:

- Motor A (Left): IN1 (D5), IN2 (D6), ENA (D9 PWM)

- Motor B (Right): IN3 (D7), IN4 (D8), ENB (D10 PWM)

- Bluetooth: TX (Arduino RX), RX (Arduino TX via voltage divider), VCC (5V), GND (Common)

## 5. Operational Logic & Software

The firmware is developed using the Arduino IDE. The logic flow follows an interrupt-driven style communication:

- Initialization: Set Baud Rate to 9600 and define motor pins as OUTPUT.

- Listening: The code constantly polls the Serial. Available() buffer for incoming ASCII characters from the mobile app.

- Decoding:

- 'F' -> Forward (IN1:H, IN2:L, IN3:H, IN4:L)

- 'B' -> Backward (IN1:L, IN2:H, IN3:L, IN4:H)

- 'L' -> Left Turn (Left Motors Back, Right Motors Forward)

- 'R' -> Right Turn (Left Motors Forward, Right Motors Back)

- 'S' -> Stop (All pins Low)

- Execution: Apply the state to digital pins to drive the H-Bridge.

## 6. Performance Analysis

- Drive Traction: The 4WD system provides a significant advantage in surface grip and torque distribution compared to 2WD models.

- Responsiveness: Average latency from mobile command to physical movement is measured under 100ms.

- Power Efficiency: The use of 18650 batteries ensures high discharge rates, preventing voltage "brown outs" when the motors start from a stall.

## 7. Maintenance & Troubleshooting

To maintain optimal performance, the following maintenance schedule is recommended:

- Voltage Calibration: Ensure batteries are charged above 10V to maintain the L298N regulator efficiency.

- Wiring Integrity: Check the screw terminals on the L298N periodically, as motor vibrations can loosen connections.

- Software Debugging: If the car moves in reverse, swap the motor output leads (Out1/Out2) instead of changing the code logic.

## 8. Conclusion & Future Roadmap

The 4WD Arduino Robot Car serves as a foundational platform for advanced mechatronics. Future versions will incorporate:

- Autonomous Obstacle Avoidance: Via HC-SR04 sensors.

- PID Speed Control: Using encoders for precise straight-line movement.

- Vision Integration: Using a serial camera or ESP32-CAM for first-person view (FPV) driving.

- Project Lead: Frais Asghar

- Engineering Domain: Robotics & Embedded Systems

- Date: February 2026